| Department | : | Quality Control |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Prepared by | : |  |  |  |  |
| Department | : | Quality Control |  |  |  |
| Reviewed by | : |  |  |  |  |
| Department | : | Research & Development |  |  |  |
| Reviewed by | : |  |  |  |  |
| Department | : | Analytical Quality Assurance |  |  |  |
| Reviewed by | : |  |  |  |  |
| Department | : | Quality Assurance |  |  |  |
| Approved by | : |  |  |  |  |
|  |  | Name | Signature | Designation | Date |

| Sr. No. | Tests | Standard & Limit | Results |
| --- | --- | --- | --- |
| 1.0 | Description | A white to off white powder. |  |
| 2.0 | Solubility | Freely soluble in water. |  |
| 3.0 | Identification |  |  |
|  | A).By IR | The IR spectrum of sample should be concordant with IR spectrum of working standard of 2-Deoxy -D- glucose. |  |
|  | B). By HPLC | The principal peak in the chromatogram obtained with the test solution corresponds to that in the chromatogram obtained with the reference solution in the Assay. |  |
| 4.0 | Water content | Not more than 1.0 % |  |
| 5.0 | Specific Optical Rotation | +44.0° to +48.0° |  |
| 6.0 | Sulphated Ash | Not more than 0.2 % |  |
| 7.0 | Related Substances (By HPLC) |  |  |
|  | Any secondary peak | Not more than 0.5% |  |
|  | Sum of all secondary peak | Not more than 2.0 % |  |
| 8.0 | Assay (By HPLC) on anhydrous basis | NLT 97.0% and NMT 103.0% |  |
| 9.0 | Residual Solvents |  |  |
|  | Toluene | NMT 890 ppm |  |
|  | Methanol | NMT 3000 ppm |  |
|  | Iso Propyl Alcohol | NMT 5000 ppm |  |
|  | Acetone | NMT 5000 ppm |  |

Remarks: Sample complies / does not comply as per above specification.

| Prepared By/Date | Checked By/Date | Approved By/Date |
| --- | --- | --- |

| 1.0 | DESCRIPTION |
| --- | --- |
|  | Instrument: |
|  | Analytical Balance ID No: I-__________; Due date of Calibration: ______________. |
|  | Procedure: |
|  | Take about ____________(2 to 3 g) of test sample in a dry petri dish, spread it by spatula and observe under suitable light for its colour and presence of any extraneous matter. |
|  | Observation: |
|  | [Limit: A white to off white powder.] |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |
| 2.0 | SOLUBILITY |
|  | Instruments: |
|  | Analytical balance ID. No.: I-________ Due date of calibration: _______________ |
|  | Procedure: |
|  | Solvent; Solubility profile; In g/mL; Observation. Water; Freely soluble; ___________gm in 1to 10 ml (About 1.0g.) in _____ ml. |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |
| 3.0 | IDENTIFICATION |
| A | BY IR |
|  | Instruments |
|  | Analytical balance ID No: I-__________; Due date of Calibration: ______________. |
|  | FTIR ID No: I-__________; Due date of Calibration: ______________. |
|  | IR conditions: |
|  | Parameter; Standard condition; Actual performed. Scan range; Between 4000 cm-1 and 400 cm-1. |
|  | For Standard: WS No.: ____________ Due date of WS: ____________ |
|  | Take approx __________ (50 mg to 100 mg) spread the standard on the ATR Approx sample holder and scan the standard. |
|  | For Sample: |
|  | Take approx __________ (50 mg to 100 mg) spread the sample on the ATR Approx sample holder and scan the sample. |
|  | Procedure: Serially analyze the standard and sample compare the sample spectrum with working standard spectrum. |
|  | Note: IR spectrum of the test sample should be compared with the spectrum of working standard stored in the library or should be freshly prepared for comparison. |
|  | Observation: |
|  | [Limit: The IR spectrum of sample should be concordant with IR spectrum of working standard of 2-Deoxy -D- glucose.] |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |
| B | By HPLC |
|  | Instruments |
|  | Analytical balance ID No: I-__________; Due date of Calibration: ______________. |
|  | HPLC ID No: I-__________; Due date of Calibration: ______________. |
|  | Procedure: Refer test No.8.0 |
|  | Observation: |
|  | [Limit: The principal peak in the chromatogram obtained with the test solution corresponds to that in the chromatogram obtained with the reference solution in the Assay.] |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |
| 4.0 | WATER CONTENT |
|  | Instruments |
|  | Analytical balance ID No: I-__________; Due date of Calibration: ______________. |
|  | Karl Fischer Apparatus ID No: I-__________; Due date of Calibration: ______________. |
|  | Reagents: |
|  | Karl Fischer reagent (AR/LR or Merck grade) |
|  | Methanol (AR/LR grade) |
|  | Procedure: |
|  | Place enough Methanol about 35 to 40 ml in the titration vessel and neutralize the water content present in the methanol with Karl Fischer reagent. Use this neutralized methanol for analysis. Accurately weigh and transfer quickly about ____________(0.2 g) of the sample in to the titration vessel. Titrate to the electrometric end point. |
|  | Calculation: |
|  | Water content =; V X FX 100. W X 1000. |
|  | Where, |
|  | V = Volume of the Karl Fischer reagent consumed in ml. |
|  | F= Karl Fischer reagent in mg/ml. |
|  | W = Weight of sample taken, in g. |
|  | Water content =___________________________ |
|  | Water content = ______________ |
|  | Observation; Limit. Not more than 1.0 %. |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |
| 5.0 | SPECIFIC OPTICAL ROTATION |
|  | Instruments |
|  | Analytical balance ID No: I-__________; Due date of Calibration: ______________. |
|  | Polarimeter ID No: I-__________; Due date of Calibration: ______________. |
|  | Solutions / Chemicals: |
|  | Water |
|  | Sample preparation: |
|  | Weigh accurately and transfer about ___________(1.0 gm) of sample in ________(100 ml) of clean and dry volumetric flask and add water, dissolve the sample and make up the volume with water. |
|  | Procedure: |
|  | Measure the Optical rotation of above sample solution with Sodium D-line (589 nm) at a temperature of 250C using methanol as blank. Calculate the Specific optical rotation and record the Optical value observed with mean of five reading. |
|  | Calculation: |
|  | Specific Optical rotation =; 100 x Mean of sample reading. Length of cell x Weight of sample in g. |
|  | Sample Reading. 1; 2; 3; 4; 5; Mean. |
|  | Specific Optical rotation = __________________________________ |
|  | Specific Optical rotation = ___________ |
|  | Observation; Limit. +44.0° to +48.0°. |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |
| 6.0 | SULPHATED ASH |
|  | Instruments |
|  | Analytical balance ID No: I-__________; Due date of Calibration: ______________. |
|  | Muffle Furnace ID No: I-__________; Due date of Calibration: ______________. |
|  | Reagents: |
|  | Sulphuric acid (AR/LR grade) |
|  | Procedure: |
|  | Dry the clean crucible in muffle furnace at ________ (800° ± 25° C) for ______(30 min), cool in a desiccator for _______ (30 min). Weigh the crucible and note the weight as W1. Accurately weigh and transfer about 1.0 g of sample into crucible and note the weight as W2. Moisten the sample with 1 mL of sulphuric acid and heat gently at as low a temperature as practicable until the sample is thoroughly charred. After cooling, moisten the residue with a small amount of sulphuric acid, heat gently until white fumes are no longer evolved. Ignite at _________ (800° ± 25° C) until the residue is completely incinerated. Allow the crucible to cool in a desiccator over silica gel and weigh it. Further heat the crucible at ________ (800° ± 25° C) for ________ (30 min). Cool and weigh it. Note the weight as W3. Repeat the procedure until two successive weighing do not difference by more than 0.0005 g. |
|  | Weight of the crucible (W1); __________ g. Weight of the crucible + sample (W2); __________ g. Weight of the sample (W2 –W1); __________ g (About 1.0). After ignition;. Weight of the crucible + residue /Ash (W3); 1.__________ g 2.__________ g. Weight of the residue/Ash (W3 – W1); __________ g. |
|  | Calculation: |
|  | (W3 – W1) % Sulphated Ash = ------------------ X 100 (W2 – W1) |
|  | Where, |
|  | W1 = Weight in g of the crucible. |
|  | W2 = Weight in g of the crucible with sample before ignition. |
|  | W3 = Weight in g of the crucible with ash after ignition. |
|  | % Sulphated Ash = _________________________ |
|  | % Sulphated Ash = ___________ |
|  | Observation; Limit. Not more than 0.2%. |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |
| 7.0 | RELATED SUBSTANCES (BY HPLC) |
|  | Instruments |
|  | Analytical balance ID No: I-__________; Due date of Calibration: ______________. |
|  | HPLC ID No: I-__________; Due date of Calibration: ______________. |
|  | Solution Preparation: |
|  | Mobile Phase preparation: |
|  | A mixture of _______(12 volumes) of water and __________(88 volumes) of Acetonitrile. |
|  | Diluent: Water : Acetonitrile ______________:____________(50:50) |
|  | Test solution: |
|  | Accurately weigh about ______________(1.0 g) of sample in ________(100 ml) volumetric flask, dissolve and dilute up to the mark with diluent. |
|  | Reference solution (a): |
|  | Accurately weigh about ____________(1.0 g) of standard in _________(100 ml) volumetric flask, dissolve and dilute up to the mark with diluent. |
|  | Reference solution (b): |
|  | Dilute _________(1 ml) of reference solution (a) in ________(100 ml) volumetric flask, dissolve and dilute up to the mark with diluent. |
|  | Chromatographic conditions: |
|  | Parameter; Standard Condition; Actual condition. Column; Shodex HILICpak VG-50 4E (250mm x 4.6 mm). Flow rate; 1.0 ml/ min. Injection volume; 10 µL loop injector. Detector; Refractive index. |
|  | ORDER OF INJECTIONS |
|  | Sr. No; Solution Name; No. of Injections. 1.0; Blank; 01. 2.0; Reference solution (a); 01. 3.0; Reference solution (b); 03. 4.0; Test solution; 01. 5.0; Reference solution (b) BKT; 01. |
|  | System suitability : |
|  | - Tailing factor is ___________(NMT 2.0) in reference solution(a). - Theoretical plates ____________(NLT 2000) in reference solution(a) |
|  | Calculation: |
|  | Any Secondary peak = Area of maximum unknown peak in test solution x 1.0 Area of Reference solution (b) |
|  | Any Secondary peak =______________________________ |
|  | Any Secondary peak =_____________ |
|  | Sum of all secondary peak = Area of Unknown peak in test solution x 1.0 Area of Reference solution (b) |
|  | Sum of all secondary peak =________________________________ |
|  | Sum of all secondary peak =____________ |
|  | Impurity; Observation; Limit. Any secondary peak; Not more than 0.5%. Sum of all secondary peak; Not more than 2.0 %. |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |
| 8.0 | ASSAY (BY HPLC) ON ANHYDROUS BASIS |
|  | Instruments |
|  | Analytical balance ID No: I-__________; Due date of Calibration: ______________. |
|  | HPLC ID No: I-__________; Due date of Calibration: ______________. |
|  | Solution Preparation: |
|  | Mobile Phase preparation: |
|  | A mixture of ________(12 volumes) of water and _________(88 volumes) of Acetonitrile. |
|  | Diluent: Water : Acetonitrile __________:________________(50:50) |
|  | Test solution: |
|  | Accurately weigh about _____________(0.2 g) of sample in ________(100 ml) volumetric flask, dissolve and dilute up to the mark with diluent. |
|  | Reference solution (a): |
|  | Accurately weigh about ___________(0.2 g) of standard in ________(100 ml) volumetric flask, dissolve and dilute up to the mark with diluent. |
|  | Chromatographic conditions: |
|  | Parameter; Standard Condition; Actual condition. Column; Shodex HILICpak VG-50 4E (250mm x 4.6 mm). Flow rate; 1.0 ml/ min. Injection volume; 10 µL loop injector. Detector; Refractive index. |
|  | Procedure: |
|  | ORDER OF INJECTIONS |
|  | Sr. No; Solution Name; No. of Injections. 1.0; Blank; 01. 2.0; Reference solution; 05. 3.0; Test solution; 02. 4.0; Reference solution BKT; 01. |
|  | % RSD for replicate injections ___________(should be more than 2.0%) in Reference solution. |
|  | Calculation: |
|  | Assay = Area of test solution x weight of standard x P x 100 Area of standard solution x weight of sample x (100-M/C) |
|  | Assay =____________________________________ |
|  | Assay = ________________ |
|  | Observation; Limit. NLT 97.0% and NMT 103.0%. |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |
| 9.0 | RESIDUAL SOLVENTS |
|  | Instruments |
|  | Analytical balance ID No: I-__________; Due date of Calibration: ______________. |
|  | GC with FID ID No: I-__________; Due date of Calibration: ______________. |
|  | Chemicals/ Solvents/Solutions: |
|  | Methanol (AR/HPLC/GC Grade) |
|  | Toluene (AR/HPLC/GC Grade) |
|  | Iso Propyl Alcohol (AR/HPLC/GC Grade) |
|  | DMSO (AR/HPLC/GC Grade) |
|  | Acetone (AR/HPLC/GC Grade) |
|  | Chromatographic conditions: |
|  | Parameter; Standard condition; Actual condition. Column; DB-624 (30 m x 0.53 mm), 3.0 µm or equivalent. Detector; FID. Carrier gas; Nitrogen. Spit Ratio; 1:20. Column flow; 2.02 ml/min. Temperature programming:. Initial Temp.; 42°C. Initial hold time; 5 min.. Rate of Temp.; 20°C/min.. Final Temp.; 200°C. Final hold time; 5.0 min.. Detector Temp; 240°C. Stop time; 18 min.. Head space condition:. Oven Temperature; 85.0°C. Sample Line Temp.; 100.0°C. Transfer Line Temp.; 140.0°C. Shaking Lavel; 3. Multi Injection Count; 1. Pressuring Gas Pressure; 50.0Kpa. Equilibrating time; 30.00 min. Pressurizing Time; 2.00 min. Pressure Equilib Time; 0.10 min. Load Time; 0.50 min. Load Equilib time; 0.10 min. Injection Time; 1.0 min. Needle Flush time; 8.0 min. GC cycle time; 40.00 min. |
|  | Test solution preparation: |
|  | Accurately weigh and transfer about ___________(500 mg) of sample in _______(5 ml) of Dimethyl sulpoxide in a 20 ml headspace vial. |
|  | Reference stock solution: |
|  | Accurately weigh and transfer about ___________(89.0 mg) of toluene in ________(100 ml) volumetric flask , dissolve and dilute up to the mark with Dimethyl sulphoxide. |
|  | Reference solution: |
|  | Accurately weigh and transfer about __________(50 mg) of isopropyl alcohol ,__________(50 mg) of Acetone and ___________(30 mg) of methanol in ________(100 ml) volumetric flask, add _______(10 ml) of reference stock solution and dissolve and dilute up to the mark with Dimethyl sulphoxide. Take 5 ml in a 20 ml head space vial. |
|  | Procedure: |
|  | Blank, Reference solution and test solution and run the chromatogram. |
|  | System suitability: |
|  | % RSD of the average area obtained for Isopropyl alcohol ,Methanol ,Acetone and Toluene for the standard solution should not be more than 15 %. |
|  | Calculation: |
|  | Toluene = Area of Test sample x 89 x 10 x 5 x 106 Area of Standard x 100 x 100 x 500 |
|  | Toluene = ___________________________ |
|  | Toluene = _________ |
|  | Methanol = Area of Test sample x 30 x 5 x106 Area of Standard x 100 x 500 |
|  | Methanol =___________________________ |
|  | Methanol =____________ |
|  | Iso propyl alcohol = Area of Test sample x 50 x 5 x106 Area of Standard x 100 x 500 |
|  | Iso propyl alcohol =______________________________ |
|  | Iso propyl alcohol =___________ |
|  | Acetone = Area of Test sample x 50 x 5 x 106 Area of Standard x 100 x 500 |
|  | Acetone =___________________________ |
|  | Acetone =_____________ |
|  | Observation; Limit. Toluene; NMT 890 ppm. Methanol; NMT 3000 ppm. Iso Propyl Alcohol; NMT 5000 ppm. Acetone; NMT 5000 ppm. |
|  | Result: Complies / does not comply |
|  | Analyzed by / Date: _______________; Checked by /Date: ______________. |