| Department | : | Quality Control |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Prepared by | : |  |  |  |  |
| Department | : | Quality Control |  |  |  |
| Reviewed by | : |  |  |  |  |
| Department | : | Research & Development |  |  |  |
| Reviewed by | : |  |  |  |  |
| Department | : | Analytical Quality Assurance |  |  |  |
| Reviewed by | : |  |  |  |  |
| Department | : | Quality Assurance |  |  |  |
| Approved by | : |  |  |  |  |
|  |  | Name | Signature | Designation | Date |

| 1. | Molecular Formula: C6H12O5 |  |
| --- | --- | --- |
| 2. | Molecular Weight: 164.2 g/mol |  |
| 3. | Structural Formula: |  |
| 4. | Sampling Plan: Draw the sample from each and every container that means 100 % sampling should be done for finish materials |  |
| 5. | Sample to be drawn from: Draw the sample from top and bottom layers using long handle sampling spatula / sampling rod. |  |
| 6. | Sample to be drawn with: Use SS sampling rod / spatula having long handle. |  |
| 7. | Sample Size: About 10 g sample from each bag to collect about 200 g sample. |  |
| 8. | Reserve Sample: 100 g |  |
| 9. | Storage Condition:Store protected from moisture. |  |
| 10. | Standard Pack Size: 25 / 30/ 50 kg packed in Fiber or HDPE Drum |  |
| 11. | Re-test Period: One year |  |
| 12. | Reference: IH |  |
| 13. | Lot wise analysis plant: Not Applicable |  |
| 14. | Hazardous and precautions: use nose mask, hand gloves while sampling, and reseal the Container after sampling. |  |

SPECIFICATION

| Sr. No. | Tests | Standard & Limit | Reference |
| --- | --- | --- | --- |
| *1.0 | Description | A white to off white powder. | IH |
| 2.0 | Solubility | Freely soluble in water. | IH |
| 3.0 | Identification |  |  |
|  | A).By IR* | The IR spectrum of sample should be concordant with IR spectrum of working standard of 2-Deoxy -D- glucose. | IH |
|  | B). By HPLC | The principal peak in the chromatogram obtained with the test solution corresponds to that in the chromatogram obtained with the reference solution in the Assay. | IH |
| 4.0* | Water content | Not more than 1.0 % | IH |
| 5.0 | Specific Optical Rotation | +44.0° to +48.0° | IH |
| 6.0 | Sulphated Ash | Not more than 0.2 % | IH |
| 7.0* | Related Substances (By HPLC) |  |  |
|  | Any secondary peak | Not more than 0.5% | IH |
|  | Sum of all secondary peak | Not more than 2.0 % | IH |
| 8.0* | Assay (By HPLC) on anhydrous basis | NLT 97.0% and NMT 103.0% | IH |
| 9.0 | Residual Solvents |  |  |
|  | Toluene | NMT 890 ppm | IH |
|  | Methanol | NMT 3000 ppm |  |
|  | Iso Propyl Alcohol | NMT 5000 ppm |  |
|  | Acetone | NMT 5000 ppm |  |

Note: * Stability indication test parameter

| Method of Analysis |  |
| --- | --- |
| 1.0 | DESCRIPTION |
|  | Instrument |
|  | Analytical Balance |
|  | Procedure: |
|  | # Take about 2 to 3 g of test sample in a dry petri dish, spread it by spatula and observe under suitable light for its colour and presence of any extraneous matter. |
|  | [Limit: A white to off white powder.] |
| 2.0 | SOLUBILITY |
|  | Instrument: |
|  | Analytical Balance |
|  | Freely soluble in water |
|  | Take about 1 g of sample in a test tube and add 1 to 10 ml of water. shake well and observe the solubility. |
|  | [Limit: Freely soluble in water.] |
| 3.0 | IDENTIFICATION |
| A | BY IR |
|  | Instruments: |
|  | Fourier Transform Infrared Spectrometer |
|  | Analytical Balance |
|  | Method of Analysis |
|  | Conditions: |
|  | Scan between 4000 cm-1 and 400 cm-1 |
|  | Standard: Take approx 50 mg to 100 mg spread the standard on the ATR Approx sample holder and scan the standard. |
|  | Sample: Take approx 50 mg to 100 mg spread the sample on the ATR Approx sample holder and Scan the Sample. |
|  | Procedure: Serially analyze the standard and sample compare the sample spectrum with working standard spectrum. |
|  | Note: IR spectrum of the test sample should be compared with the spectrum of working standard stored in the library or should be freshly prepared for comparison. |
|  | [Limit: The IR spectrum of sample should be concordant with IR spectrum of working standard of 2-Deoxy -D- glucose.] |
| B | By HPLC |
|  | Instruments: |
|  | HPLC |
|  | Analytical Balance |
|  | Procedure: Refer test No.8.0 |
|  | [Limit: The principal peak in the chromatogram obtained with the test solution corresponds to that in the chromatogram obtained with the reference solution in the Assay.] |
|  | Method of Analysis |
| 4.0 | WATER CONTENT |
|  | Instruments: |
|  | Analytical balance |
|  | Karl Fischer Apparatus |
|  | Reagents: |
|  | Karl Fischer reagent (AR/LR or Merck grade) |
|  | Methanol (AR/LR grade) |
|  | Procedure: |
|  | Place enough Methanol about 35 to 40 ml in the titration vessel and neutralize the water content present in the methanol with Karl Fischer reagent. Use this neutralized methanol for analysis. Accurately weigh and transfer quickly about 0.2 g of the sample in to the titration vessel. Titrate to the electrometric end point. |
|  | Calculation: |
|  | Water =; V X FX 100. W X 1000. |
|  | Where, |
|  | V = Volume of the Karl Fischer reagent consumed in ml. |
|  | F= Karl Fischer reagent in mg/ml. |
|  | W = Weight of sample taken, in g. |
|  | [Limit: Not more than 1.0 %] |
|  | Method of Analysis |
| 5.0 | SPECIFIC OPTICAL ROTATION |
|  | Instruments |
|  | Analytical balance |
|  | Polarimeter |
|  | Solutions / Chemicals: |
|  | Water |
|  | Sample preparation: |
|  | Weigh accurately and transfer about 1.0 gm of sample in 100 ml of clean and dry volumetric flask and add water, dissolve the sample and make up the volume with water. |
|  | Procedure: |
|  | Measure the Optical rotation of above sample solution with Sodium D-line (589 nm) at a temperature of 250C using methanol as blank. Calculate the Specific optical rotation and record the Optical value observed with mean of five reading. |
|  | Calculation: |
|  | Specific Optical rotation =; 100 x Mean of sample reading. Length of cell x Weight of sample in g. |
|  | [Limit: +44.0° to +48.0°] |
|  | Method of Analysis |
| 6.0 | SULFATED ASH |
|  | Instruments |
|  | Analytical balance |
|  | Muffle furnace |
|  | Reagents / chemicals / solvents: |
|  | Sulphuric acid |
|  | Procedure: |
|  | Dry the clean crucible in muffle furnace at 800° ± 25° C for 30 min, cool in a desiccator for 30 min. Weigh the crucible and note the weight as W1. Accurately weigh and transfer about 1.0 g of sample into crucible and note the weight as W2. Moisten the sample with 1 mL of sulphuric acid and heat gently at as low a temperature as practicable until the sample is thoroughly charred. After cooling, moisten the residue with a small amount of sulphuric acid, heat gently until white fumes are no longer evolved. Ignite at 800° ± 25° C until the residue is completely incinerated. Allow the crucible to cool in a desiccator over silica gel and weigh it. Further heat the crucible at 800° ± 25° C for 30 min. Cool and weigh it. Note the weight as W3. Repeat the procedure until two successive weighing do not difference by more than 0.0005 g. |
|  | Calculation: |
|  | % Sulfated Ash =; (W3 – W1); x 100. (W2 – W1). |
|  | Method of Analysis |
|  | Where, |
|  | W1= Weight in g of the crucible. W2= Weight in g of the crucible with sample before ignition. |
|  | W3= Weight in g of the crucible with residue after ignition. |
|  | [Limit: Not more than 0.2 %] |
| 7.0 | RELATED SUBSTANCES (BY HPLC) |
|  | Instruments |
|  | Analytical balance |
|  | HPLC |
|  | Solution Preparation: |
|  | Mobile Phase preparation: |
|  | A mixture of 12 volumes of water and 88 volumes of Acetonitrile. |
|  | Diluent:Water : Acetonitrile (50:50) |
|  | Test solution: |
|  | Accurately weigh about 1.0 g of sample in 100 ml volumetric flask, dissolve and dilute up to the mark with diluent. |
|  | Reference solution (a): |
|  | Accurately weigh about 1.0 g of standard in 100 ml volumetric flask, dissolve and dilute up to the mark with diluent. |
|  | Method of Analysis |
|  | Reference solution (b): |
|  | Dilute 1 ml of reference solution (a) in 100 ml volumetric flask ,dissolve and dilute up to the mark with diluent. |
|  | Chromatographic conditions: |
|  | Parameter; Standard Condition. Column; Shodex HILICpak VG-50 4E (250mm x 4.6 mm). Flow rate; 1.0 ml/ min. Injection volume; 10 µL loop injector. Detector; Refractive index. |
|  | ORDER OF INJECTIONS |
|  | Sr. No; Solution Name; No. of Injections. 1.0; Blank; 01. 2.0; Reference solution (a); 01. 3.0; Reference solution (b); 03. 4.0; Test solution; 01. 5.0; Reference solution (b) BKT; 01. |
|  | System suitability : |
|  | - Tailing factor is NMT 2.0 in reference solution(a). - Theoretical plates NLT 2000 in reference solution(a) |
|  | Method of Analysis |
|  | Calculation: |
|  | Any Secondary peak = Area of maximum unknown peak in test solution x 1.0 Area of Reference solution (b) |
|  | Sum of all secondary peak = Area of Unknown peak in test solution x 1.0 Area of Reference solution (b) |
|  | Impurity; Limit. Any secondary peak; Not more than 0.5%. Sum of all secondary peak; Not more than 2.0 %. |
| 8.0 | ASSAY (BY HPLC) ON ANHYDROUS BASIS |
|  | Instruments |
|  | Analytical balance |
|  | HPLC |
|  | Solution Preparation: |
|  | Mobile Phase preparation: |
|  | A mixture of 12 volumes of water and 88 volumes of Acetonitrile. |
|  | Diluent:Water : Acetonitrile (50:50) |
|  | Test solution: |
|  | Accurately weigh about 0.2 g of sample in 100 ml volumetric flask, dissolve and dilute up to the mark with diluent. |
|  | Method of Analysis |
|  | Reference solution : |
|  | Accurately weigh about 0.2 g of standard in 100 ml volumetric flask, dissolve and dilute up to the mark with diluent. |
|  | Chromatographic conditions: |
|  | Parameter; Standard Condition. Column; Shodex HILICpak VG-50 4E (250mm x 4.6 mm). Flow rate; 1.0 ml/ min. Injection volume; 10 µL loop injector. Detector; Refractive index. |
|  | Procedure: |
|  | ORDER OF INJECTIONS |
|  | Sr. No; Solution Name; No. of Injections. 1.0; Blank; 01. 2.0; Reference solution; 05. 3.0; Test solution; 02. 4.0; Reference solution BKT; 01. |
|  | % RSD for replicate injections should be more than 2.0% in Reference solution. |
|  | Method of Analysis |
|  | Calculation: |
|  | Assay = Area of test solution x weight of standard x P x 100 Area of standard solution x weight of sample x (100-M/C) |
|  | [Limit: NLT 97.0% and NMT 103.0%] |
| 9.0 | RESIDUAL SOLVENTS |
|  | Instruments |
|  | Analytical balance |
|  | GC with FID |
|  | Chemicals/ Solvents/Solutions: |
|  | Methanol (AR/HPLC/GC Grade) |
|  | Toluene (AR/HPLC/GC Grade) |
|  | Iso Propyl Alcohol (AR/HPLC/GC Grade) |
|  | DMSO (AR/HPLC/GC Grade) |
|  | Acetone (AR/HPLC/GC Grade) |
|  | Chromatographic conditions: |
|  | Parameter; Standard condition. Column; DB-624 (30 m x 0.53 mm), 3.0 µm or equivalent. Detector; FID. |
|  | Method of Analysis |
|  | Parameter; Standard condition. Carrier gas; Nitrogen. Spit Ratio; 1:20. Column flow; 2.02 ml/min. Temperature programming:. Initial Temp.; 42°C. Initial hold time; 5 min.. Rate of Temp.; 20°C/min.. Final Temp.; 200°C. Final hold time; 5.0 min.. Detector Temp; 240°C. Stop time; 18 min.. Head space condition:. Oven Temperature; 85.0°C. Sample Line Temp.; 100.0°C. Transfer Line Temp.; 140.0°C. Shaking Lavel; 3. Multi Injection Count; 1. Pressuring Gas Pressure; 50.0Kpa. Equilibrating time; 30.00 min. Pressurizing Time; 2.00 min. Pressure Equilib Time; 0.10 min. Load Time; 0.50 min. Load Equilib time; 0.10 min. Injection Time; 1.0 min. Needle Flush time; 8.0 min. GC cycle time; 40.00 min. |
|  | Method of Analysis |
|  | Test solution preparation: |
|  | Accurately weigh and transfer about 500 mg of sample in 5 ml of Dimethyl sulpoxide in a 20 ml headspace vial. |
|  | Reference stock solution: |
|  | Accurately weigh and transfer about 89.0 mg of toluene in 100 ml volumetric flask , dissolve and dilute up to the mark with Dimethyl sulphoxide. |
|  | Reference solution: |
|  | Accurately weigh and transfer about 50 mg of isopropyl alcohol ,50 mg of Acetone and 30 mg of methanol in 100 ml volumetric flask, add 10 ml of reference stock solution and dissolve and dilute up to the mark with Dimethyl sulphoxide.Take 5 ml in a 20 ml head space vial. |
|  | Procedure: |
|  | Blank, Reference solution and test solution and run the chromatogram. |
|  | System suitability: |
|  | % RSD of the average area obtained for Isopropyl alcohol ,Methanol ,Acetone and Toluene for the standard solution should not be more than 15 %. |
|  | Calculation: |
|  | Toluene = Area of Test sample x 89 x 10 x 5 x 106 Area of Standard x 100 x 100 x 500 |
|  | Method of Analysis |
|  | Methanol = Area of Test sample x 30 x 5 x 106 Area of Standard x 100 x 500 |
|  | Iso propyl alcohol = Area of Test sample x 50 x 5 x 106 Area of Standard x 100 x 500 |
|  | Acetone = Area of Test sample x 50 x 5 x 106 Area of Standard x 100 x 500 |
|  | Limit. Toluene; NMT 890 ppm. Methanol; NMT 3000 ppm. Iso Propyl Alcohol; NMT 5000 ppm. Acetone; NMT 5000 ppm. |

HISTORY OF CHANGES

| Revision No. | Effective Date | Summary of Changes |  |
| --- | --- | --- | --- |
| 01 |  | i | Finished Product specification 2-Deoxy-D-Glucose IH is Prepared with reference change control No: CCP/21/144. |