- DESCRIPTION Specification: White crystalline powder. Observation: __________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- IDENTIFICATION

- Phosphocreatine (IR spectrum) Instrument ID: _______________________ Calibration due on: _______________ Specification: Conforms to the standard. Observation: __________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- Creatine Specification: Red coloring should appear. α-Naphthol Solution. Dissolve _______ (0.10 g) of α -naphthol in _______ (3 mL) of a 15% w/v solution of sodium hydroxide and dilute to _______ (100 mL) with water. Prepare immediately before use. Procedure. _______ (2 ml) of an aqueous solution containing ______ (10 mg) of creatine phosphate is heated in a boiling water bath for 10 minutes, ______ (2 ml) of α -Naphthol and diacetyl solution are added. Observation: __________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- Phosphorus Specification: Blue coloring should appear. Reagent A. _______ (5g) of ammonium molybdate is placed in a _____ (100 ml) volumetric flask, dissolved in a 1 M solution of sulfuric acid and the volume in the flask is brought to the mark with the same solvent. Reagent B. _______ (200 mg) of p-methylaminophenol sulfate is dissolved in _______ (100 ml) of water and _______ (20g) of sodium sulfate is added. The reagent should be stored in tightly closed containers and used for 1 month.

| Procedure. _______ (10 mg) of creatine phosphate is dissolved in _________ (25 ml) of 0.5 M sulfuric acid, heated in a boiling water bath for 10 minutes, cooled and added ml of reagent A and ml of reagent B, stirred and left for a while. Observation: __________________________________________________________________________ |  |
| --- | --- |
| Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply |  |
| - Sodium Specification: Flame staining on yellow. Procedure. When the drug is introduced into the flame of the burner, a yellow staining appears, characteristic of sodium. Observation: ________________________________________________________________________ |  |
| Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply |  |
| - OMOGENEITY BY MASS OF VIAL CONTENTS Specification: ± 10 %. №; m1; m2; m3; №; m1; m2; m3. 1; 11. 2; 12. 3; 13. 4; 14. 5; 15. 6; 16. 7; 17. 8; 18. 9; 19. 10; 20. Δm=m320 = 20 = g. |
| %= g* 100 g; %= g* 100 g. %= g* 100 g; %= g* 100 g. %= g* 100 g; %= g* 100 g. %= g* 100 g; %= g* 100 g. %= g* 100 g; %= g* 100 g. %= g* 100 g; %= g* 100 g. %= g* 100 g; %= g* 100 g. %= g* 100 g; %= g* 100 g. %= g* 100 g; %= g* 100 g. %= g* 100 g; %= g* 100 g. Observation: _________________________________________________________________________ |
| Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply |
| - TRANSPARENCY Specification: The solution must be transparent (2.2.1). Observation: _________________________________________________________________________ |
| Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply |
| - pH value Instrument ID: _______________________ Calibration due on: _______________ Specification: 8.0 - 9.0 Observation: _________________________________________________________________________ |
| Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply |

- PARTICULATE CONTAMINATION

- Visible Particles Specification: There must be no visible particles Observation: _________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- Invisible Particles Instrument ID: _______________________ Calibration due on: _______________ Specification: NMT 10 µm 6000 per container; NMT 25 µm 600 per container. Observation: ________________________________________________________________________ _____________________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- SOLUBILITY Specification: The contents of one bottle should completely dissolve in a bottle with a solvent to form a transparent solution practically free of foreign particles. Observation: _________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- IMPURITIES Instrument ID: _______________________ Calibration due on: _______________ Specification: Sum of impurities: NMT 5 % 0.005 M tetrabutylammonium phosphate solution. _______ (1.7 g) of tetrabutylammonium hydrophosphate is placed in a measuring flask with a capacity of _______ (1000 ml), dissolved in 900 ml of water, adjusted pH - ______ (7) with a solution of sodium phosphate 20% and adjusted the volume in the flask with water to the mark. Standard solution A. About ______ (5 mg) of creatine, about _______ (5 mg0 of creatinine and phosphate are placed in a _______ (100 ml) volumetric flask, dissolved in water and the volume in the flask is brought to the mark with the same solvent and mixed. Standard solution B. Pour ______ (2.0 ml) of the sample solution into a _______ (100 ml) volumetric flask, bring to the volume with water. Test solution. About _________ (50 mg) of the drug is placed in a measuring flask with a capacity of ____ (10 ml), dissolved in water and the volume in the flask is brought to the mark with the same solvent, mixed.

Standard; Test. Purity of Standard; Test weight. Standard weight; Dilution. Dilution; No of injection; Area. No of injection; Area; 1. STD A (1); 2. STD A (2); Mean. STD A (3); Retention time. STD B (1); Creatine. STD B (2); Creatinine. Resolution (NLT 2.5); Creatine phosphate. RSD (NMT 15%); Creatinine phosphate. X= Observation: ________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- WATER Instrument ID: _______________________ Calibration due on: _______________ Specification: 22.0% - 25.0%. Observation: ________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- QUANTITY (HPLC method) Instrument ID: _______________________ Calibration due on: _______________ Specification: 95.0% - 110.0% Test solution: About _______ (50 mg) (exact weight) of the drug is placed in a measuring flask with a capacity of _____ (100 ml), dissolved in water and the volume in the flask is brought to the mark with the same solvent, mixed. The solution is prepared before use. Standard solution: About _______ (50 mg) (exact weight) of creatine phosphate (company standard) is placed in a measuring flask with a capacity of ________ (100 ml), dissolved in water and the volume in the flask is brought to the mark with the same solvent and mixed. Prepare the solution before use. Mobile phase (0.1 % phosphoric acid solution): _______ (1.0 ml) of concentrated phosphoric acid is placed in a measuring flask with a capacity of _________ (1000 ml), dissolved in 900 ml of water, and adjusted the volume in the flask with water to the mark.3. Standard; Test. Purity of Standard; Test weight. Standard weight; Dilution. Dilution; No of injection; Area. No of injection; Area; 1. 1; 2. 2; Mean. 3; RSD. 4. 5; Bracketing standard. Mean; 1. RSD; 2.

X= = Observation: _________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- BACTERIAL ENDOTOXIN Specification: NMT 0.175 EU/mg. Observation: ________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- STERILITY Specification: Should be sterile Observation: ________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- PACKAGING Specification: Conforms to the approved packaging artwork Observation: ________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

- LABELLING Specification: Conforms to the approved labelling artwork Observation: ________________________________________________________________________

Analysed by: _______________________/__________ Date: _____/_____/_________ Complies/Does not comply

| Analysed By |  | Reviewed By |  |
| --- | --- | --- | --- |
| Sign/ Date |  | Sign/ Date |  |
| Designation |  | Designation |  |