| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Pharmacopeial reference | : | USP |
| Special requirement of pack | : | Material should be packed in HDPE drum. |
| Approved vendor | : | Please refer current approved vendor list. |
| Handling hazards | : | Refer Material Safety Data Sheet |
| Quantity to be sampled |  | Analysis Sample; Control sample. 50 g; 100 g. |
| Retest Period | : | 12 Months |
| Storage | : | Preserve in well closed container. Do not store above 55˚ C. |

| Sr. No. | TEST | SPECIFICATIONS |  |  |  |  | REFERENCE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Description # | White to yellowish liquid, , fatty solid to semisolid |  |  |  |  | USP |
|  | Solubility | Insoluble in water |  |  |  |  | USP |
|  | Identification Test |  |  |  |  |  |  |
|  | A) By Fatty Acid Composition | Carbon chain length | No. of double bonds |  | Percentage |  | USP |
|  |  | ≤ 12 | 0 |  | ≤ 2.5 |  |  |
|  |  | 14 | 0 |  | 0.5 – 5.9 |  |  |
|  |  | 16 | 0 |  | 39.0 –47.0 |  |  |
|  |  | 18 | 0 |  | 2.0 – 8.0 |  |  |
|  |  | 16 | 1 |  | ≤ 0.5 |  |  |
|  |  | 18 | 1 |  | 36.0 – 44.0 |  |  |
|  |  | 18 | 2 |  | 7.0 – 12.0 |  |  |
|  |  | 18 | 3 |  | ≤ 0.5 |  |  |
|  |  | ≥20 | 0 or 1 |  | ≤ 1.0 |  |  |
|  | B) Melting Range or Temperature | 30° C-40° C |  |  |  |  | USP |
|  | C) By Triglyceride Profile | The Rf values of the principal spots of the sample solution should correspond to those of the standard solution. |  |  |  |  | USP |
|  | Residue On Ignition | NMT 0.1% |  |  |  |  | USP |
|  | Alkaline Impurities | NMT 0.1 ml of 0.01 N Hydrochloric acid is required |  |  |  |  | USP |
|  | Melting Range or Temperature | 30°C - 40°C |  |  |  |  | USP |
|  | Fats And Fixed oils |  |  |  |  |  | USP |
|  | - Acid Value | Not more than 2.0 |  |  |  |  |  |
|  | - Fatty Acid Composition | Carbon chain length |  | No. of double bonds |  | Percentage |  |
|  |  | ≤ 12 |  | 0 |  | ≤ 2.5 |  |
|  |  | 14 |  | 0 |  | 0.5 – 5.9 |  |
|  |  | 16 |  | 0 |  | 39.0 –47.0 |  |
|  |  | 18 |  | 0 |  | 2.0 – 8.0 |  |
|  |  | 16 |  | 1 |  | ≤ 0.5 |  |
|  |  | 18 |  | 1 |  | 36.0 – 44.0 |  |
|  |  | 18 |  | 2 |  | 7.0 – 12.0 |  |
|  |  | 18 |  | 3 |  | ≤ 0.5 |  |
|  |  | ≥20 |  | 0 or 1 |  | ≤ 1.0 |  |
|  | - Peroxide Value | NMT 5.0 |  |  |  |  |  |
|  | - Unsaponifiable Matter | NMT 1.0 % |  |  |  |  |  |
|  | Water Determination | NMT 0.1 % |  |  |  |  | USP |

#: Re-evaluation  tests

REVISION HISTORY FOR SPECIFICATION:

| Revision No. | Revision Description | Effective Date |
| --- | --- | --- |
| 01 | Specification prepared as per Approved Change control CC/PD/23/011 |  |